# Table of Contents

Product Requirements Document: Analytics Dashboards

Executive Summary

User Journey Map

Dashboard User Journeys by Role

Data Flow to Dashboards

Dashboard Hierarchy

Stakeholder Insights

Key Feedback from Nick Luciano

Key Feedback from Jim (TJ)

Dashboard Inventory

Priority 1: Operations Dashboards

Priority 2: Analysis Dashboards

Priority 3: Experiment Dashboards

Priority 4: Digital Twin Dashboards

Dashboard Specifications

1. Reactor Operations Overview

2. Operations Log Compliance

3. Fuel Burnup Heatmap

4. Xenon Inventory (Inferred)

5. Shift Summary / Handover Report

Security Requirements

Access Tiers

Implementation

Data Refresh Patterns

Export Requirements

Supported Formats

Integration Points

From Reactor Ops Log

From Experiment Manager

From Time-Series (Iceberg)

From Models

MVP Scope (Phase 1)

In Scope

Out of Scope (Future)

Open Questions

Success Metrics

Appendix: Stakeholder Quotes Reference

# Product Requirements Document: Analytics Dashboards

Module: Superset Analytics Dashboards   Status: Draft   Last Updated: January 21, 2026   Stakeholder Input: Jim (TJ), Nick Luciano (Jan 2026)   Parent: Executive PRD

## Executive Summary

Analytics Dashboards provide visual insights into reactor operations, experiment tracking, fuel performance, and system health. Built on Apache Superset, dashboards enable both real-time monitoring (for operations staff) and historical analysis (for researchers and management). The goal is to transform raw reactor data into actionable information.

## User Journey Map

### Dashboard User Journeys by Role

### Data Flow to Dashboards

### Dashboard Hierarchy

## Stakeholder Insights

### Key Feedback from Nick Luciano

#### On Live Streaming

"We'd ideally get live streaming, but currently we just upload the data after-hours due to cost."

Design Implication: Dashboard architecture is streaming-first with batch fallbacks. Real-time is the default; batch processing handles historical aggregations and disaster recovery. See .ADR 007

#### On Security Concerns

"The public should generally not know when the reactor is at power. More specifically, we don't want live updates on the reactor status available online. A Superset login that is well protected is sufficient."

Design Implication: Dashboards require authentication. No public-facing real-time power status. Day-after data is acceptable for broader access.

#### On Calendar vs. Reality

"The calendar is used to schedule time. It is not a reflection of what actually happened."

Design Implication: Dashboard should show actual reactor data, not scheduled data. Calendar integration is low priority.

#### On Xenon Tracking

"This [xenon] cannot be measured directly, but can be correlated with critical rod heights."

Design Implication: Xenon dashboard should use inferred/calculated values, not direct measurement. Show correlation methodology.

#### On Fuel Burnup

"This [fuel burnup heatmap] would be great. Ultimately, a product of the code would be burnup values for individual fuel elements. I could see a time series of burnup being displayed."

Design Implication: Fuel burnup heatmap is high-value. Requires model integration for per-element calculations.

### Key Feedback from Jim (TJ)

#### On Operational Insights

"We can ascertain a baseline fuel 'burn up' amount when starting up from a long period of shut down. During operation, fuel 'burn up' amounts can be tracked as well."

Design Implication: Burnup tracking should show both baseline (post-shutdown) and operational (during run) values.

#### On Log Entry Counts

"I am not sure what insight could be achieved by counting the number of log entries. We could however add a 'watch type' to all entries and then gather what operations we normally perform at what hour."

Design Implication: Simple entry counts are not useful. But tagging entries by type (watch type, category) enables meaningful analysis—e.g., "What operations happen at 2 AM vs 2 PM?"

#### On Compliance Gaps

"A gap would mean that this :30 minute check was not performed when operating."

Design Implication: Gaps in mandatory checks should be visually prominent—this is a compliance indicator.

#### On Export Requirements

"Export to PDF would work, but a simple text file for archive and future proof would also work."

Design Implication: Support multiple export formats. Plain text for archival longevity.

## Dashboard Inventory

Based on stakeholder feedback, prioritized:

### Priority 1: Operations Dashboards

| Dashboard | Primary Users | Key Insight | Stakeholder Quote |
| --- | --- | --- | --- |
| Reactor Operations Overview | Operators, RM | Real-time (or near-real-time) reactor status | Nick: "We'd ideally get live streaming" |
| Operations Log Compliance | RM, NRC | Gaps in mandatory checks | Jim: "A gap would mean... check was not performed" |
| Shift Summary | SRO, RM | Entries by watch/shift, handover report | Jim: "add a 'watch type' to all entries" |

### Priority 2: Analysis Dashboards

| Dashboard | Primary Users | Key Insight | Stakeholder Quote |
| --- | --- | --- | --- |
| Fuel Burnup Heatmap | RM, Analysts | Per-element burnup visualization | Nick: "This would be great" |
| Xenon Inventory (Inferred) | Operators, Analysts | Xe-135 tracking via rod height correlation | Nick: "cannot be measured directly, but can be correlated" |
| Power History | Researchers | Historical power levels, trends | Standard operational need |

### Priority 3: Experiment Dashboards

| Dashboard | Primary Users | Key Insight | Stakeholder Quote |
| --- | --- | --- | --- |
| Experiment Tracking | Researchers, PIs | Sample status, beam time usage | Khiloni: correlate with reactor conditions |
| Facility Usage | RM, PIs | Which facilities are most used | Operational planning |

### Priority 4: Digital Twin Dashboards

| Dashboard | Primary Users | Key Insight | Stakeholder Quote |
| --- | --- | --- | --- |
| Model vs. Measurement | Analysts | Validation of DT predictions | Foundational DT metric |
| Model Performance | Developers | Execution times, convergence | Technical health |

## Dashboard Specifications

### 1. Reactor Operations Overview

Purpose: Real-time (or near-real-time) view of reactor status for console monitoring.

Security Note: Requires authentication. Not public-facing per Nick's security concern.

Panels:

Current Power Level (big number with trend sparkline)

Control Rod Positions (bar chart or rod diagram)

Pool Temperature (gauge with alarm thresholds)

Coolant Flow Rate (if measured)

Status Indicator (Shutdown / Startup / Operating / Scram)

Time Since Last Ops Log Entry (compliance indicator)

Data Refresh & Freshness:

Design Decision: Streaming-first with batch fallbacks. See ADR 007

| Data Type | Target Latency | Implementation |
| --- | --- | --- |
| Ops Log entries | <500ms | WebSocket push |
| 30-min rule timer | <100ms | Real-time sync |
| Reactor status | <2s | Event-driven update |
| Historical aggregations | Minutes | Batch job (Dagster) |

Freshness Indicator: Live is the default — users assume data is current. Warnings appear only when streaming is degraded:

🟢 Live — default state, no indicator needed

⚠️ Degraded — streaming delayed, showing last known values

🔴 Stale — connection lost or >5 min behind

Mockup:

┌────────────────────────────────────────────────────────────────┐
│  REACTOR OPERATIONS OVERVIEW                                   │
│  🟡 Last Updated: 5 min ago              [⟳ Refresh] [Settings]│
├────────────────────────────────────────────────────────────────┤
│                                                                │
│  ┌──────────────┐  ┌──────────────┐  ┌──────────────┐         │
│  │   POWER      │  │   STATUS     │  │  POOL TEMP   │         │
│  │   950 kW     │  │  OPERATING   │  │   85°F       │         │
│  │   ▲ +50kW    │  │   ● ● ● ●    │  │   ───────    │         │
│  └──────────────┘  └──────────────┘  └──────────────┘         │
│                                                                │
│  ┌────────────────────────────────────────────────────────┐   │
│  │  CONTROL RODS                                          │   │
│  │  ████████░░ A: 72%  ████████░░ B: 68%  ████░░░░░ C: 41%│   │
│  └────────────────────────────────────────────────────────┘   │
│                                                                │
│  ┌────────────────────────────────────────────────────────┐   │
│  │  POWER HISTORY (Last 24h)                               │   │
│  │  1000 ─┬─────────────────────────────────────────────  │   │
│  │        │                     ████████████████████████  │   │
│  │   500 ─┼───────────█████████                           │   │
│  │        │    ███████                                     │   │
│  │     0 ─┴─────────────────────────────────────────────  │   │
│  │        00:00        06:00        12:00        18:00     │   │
│  └────────────────────────────────────────────────────────┘   │
│                                                                │
└────────────────────────────────────────────────────────────────┘

### 2. Operations Log Compliance

Purpose: Ensure mandatory 30-minute checks are being performed; surface gaps for RM review.

Panels:

Gap Indicator (big red/green: "X gaps this shift")

Timeline of Entries (dots on timeline, gaps highlighted)

Entries by Type (pie chart)

Entries by Operator (bar chart)

Entries by Watch Type (per Jim's suggestion)

Key Metric:

Gap = Operating period > 30 min without CONSOLE_CHECK entry

Mockup:

┌────────────────────────────────────────────────────────────────┐
│  OPS LOG COMPLIANCE DASHBOARD                [Shift: Day]      │
├────────────────────────────────────────────────────────────────┤
│                                                                │
│  ┌──────────────┐  ┌──────────────────────────────────────┐   │
│  │   GAPS       │  │  ENTRY TIMELINE (Today)              │   │
│  │              │  │                                      │   │
│  │    0  ✓      │  │  ●───●───●───●───●───●───●───●───●  │   │
│  │              │  │  08  09  10  11  12  13  14  15  16  │   │
│  │  This Shift  │  │                                      │   │
│  └──────────────┘  └──────────────────────────────────────┘   │
│                                                                │
│  ┌──────────────────────┐  ┌─────────────────────────────┐    │
│  │  ENTRIES BY TYPE     │  │  ENTRIES BY OPERATOR        │    │
│  │                      │  │                             │    │
│  │  Console Check: 12   │  │  J. Smith: 8                │    │
│  │  Experiment: 3       │  │  M. Jones: 4                │    │
│  │  General Note: 2     │  │  K. Lee: 5                  │    │
│  │  Maintenance: 1      │  │                             │    │
│  └──────────────────────┘  └─────────────────────────────┘    │
│                                                                │
│  ┌────────────────────────────────────────────────────────┐   │
│  │  ⚠️ GAPS IN LAST 30 DAYS                               │   │
│  │                                                        │   │
│  │  2026-01-15 14:30-15:15 (45 min gap) - Shift: Night   │   │
│  │  2026-01-08 02:00-02:45 (45 min gap) - Shift: Night   │   │
│  └────────────────────────────────────────────────────────┘   │
│                                                                │
└────────────────────────────────────────────────────────────────┘

### 3. Fuel Burnup Heatmap

Purpose: Visualize per-element fuel burnup for operational planning and regulatory reporting.

Data Source: Model output (not direct measurement). Per Nick: "a product of the code would be burnup values for individual fuel elements."

Panels:

Core Map Heatmap (top-down view, colored by burnup)

Burnup by Element (sortable table)

Burnup Trend (time series for selected element)

Baseline vs Current (per Jim: "baseline fuel 'burn up' amount when starting up from a long period of shut down")

Mockup:

┌────────────────────────────────────────────────────────────────┐
│  FUEL BURNUP VISUALIZATION                    [As of: 2026-01]│
├────────────────────────────────────────────────────────────────┤
│                                                                │
│  ┌──────────────────────────────┐  ┌────────────────────────┐ │
│  │  CORE MAP (Top-Down)         │  │  BURNUP SCALE          │ │
│  │                              │  │                        │ │
│  │      ░░░███░░░               │  │  ████ High (>50 MWd)   │ │
│  │    ░░████████░░              │  │  ▓▓▓▓ Med  (25-50)     │ │
│  │   ░███▓▓▓▓▓▓███░             │  │  ░░░░ Low  (<25 MWd)   │ │
│  │   ░██▓▓▓▓▓▓▓▓██░             │  │                        │ │
│  │   ░███▓▓▓▓▓▓███░             │  │  Click element for     │ │
│  │    ░░████████░░              │  │  detail view           │ │
│  │      ░░░███░░░               │  │                        │ │
│  │                              │  │                        │ │
│  └──────────────────────────────┘  └────────────────────────┘ │
│                                                                │
│  ┌────────────────────────────────────────────────────────┐   │
│  │  BURNUP HISTORY (Selected: Element B-4)                │   │
│  │                                                        │   │
│  │  60 MWd ─┬─────────────────────────────────────█████  │   │
│  │          │                              ███████       │   │
│  │  30 MWd ─┼────────────────███████████████            │   │
│  │          │   ██████████████                           │   │
│  │   0 MWd ─┴─────────────────────────────────────────   │   │
│  │          2022        2023        2024        2025     │   │
│  └────────────────────────────────────────────────────────┘   │
│                                                                │
└────────────────────────────────────────────────────────────────┘

### 4. Xenon Inventory (Inferred)

Purpose: Track Xe-135 buildup/decay for operational planning (startup timing, power maneuvers).

Data Source: Inferred from critical rod heights, per Nick: "cannot be measured directly, but can be correlated with critical rod heights."

Panels:

Estimated Xe-135 Inventory (time series)

Critical Rod Height Correlation (scatter plot showing relationship)

Methodology Note (explain inference approach)

Important Caveat: Display prominently that this is inferred data, not direct measurement.

### 5. Shift Summary / Handover Report

Purpose: Generate end-of-shift summary for handover to incoming crew.

Content:

Power level changes during shift

All Ops Log entries (with supplements)

Experiments conducted

Equipment issues noted

Pending items for next shift

Export: PDF and plain text per Jim's requirement.

## Security Requirements

From Nick:

"The public should generally not know when the reactor is at power."

### Access Tiers

| Tier | Access Level | Example Users |
| --- | --- | --- |
| Public | None (no public dashboards) | N/A |
| Delayed | Data from >24 hours ago | External researchers (future) |
| Real-time | Current data | Operators, RM, HP |
| Admin | Full access + configuration | System administrators |

### Implementation

All dashboards behind Superset authentication

No embedding of live dashboards in public websites

Day-after exports acceptable for external sharing

## Data Refresh Patterns

| Pattern | Latency | Use Case |
| --- | --- | --- |
| Nightly Batch | ~12 hours | Current implementation; sufficient for historical analysis |
| Near-Real-Time | ~5 min | Future; needed for operations dashboard |
| Real-Time | <1 min | Future; needed for console monitoring |

Nick's note: "We'd ideally get live streaming, but currently we just upload the data after-hours due to cost."

Design Decision: Streaming-first architecture. Real-time is the default; batch for historical aggregations and fallback. See .ADR 007

## Export Requirements

Per Jim: "Export to PDF would work, but a simple text file for archive and future proof would also work."

### Supported Formats

| Format | Notes |
| --- | --- |
| PDF | Charts rendered as images; professional formatting |
| PNG/SVG | Individual chart export |
| CSV | Underlying data for any chart |
| Plain Text | For compliance reports and archival |

## Integration Points

### From Reactor Ops Log

Entry counts by type/time

Gap detection alerts

Watch-type tagging

### From Experiment Manager

Sample status aggregations

Facility utilization

### From Time-Series (Iceberg)

Power levels, temperatures, rod positions

Historical queries for trends

### From Models

Burnup calculations

Xenon inferred values

Model vs measurement comparisons

## MVP Scope (Phase 1)

### In Scope

Reactor Operations Overview (with nightly batch data)

Operations Log Compliance (gap detection)

Power History (simple time-series charts)

Basic authentication

### Out of Scope (Future)

Real-time streaming

Fuel Burnup Heatmap (requires model integration)

Xenon Inference (requires model)

External/delayed access tier

## Open Questions

Real-time priority: How important is real-time vs cost savings of nightly batch?

Historical depth: How far back should dashboards query? (2 years per NRC, or full history?)

Alert routing: Should compliance gaps trigger email/SMS to RM?

Model integration: What's the timeline for burnup model output integration?

Watch types: What are the standard watch types to tag entries with?

## Success Metrics

| Metric | Target | Measurement |
| --- | --- | --- |
| Dashboard adoption | 80% of operators check daily | Login analytics |
| Gap detection accuracy | 100% of gaps surfaced | Audit comparison to paper logs |
| Export usage | NRC inspection prep uses exports | Staff survey |
| Insight generation | RM identifies 1+ operational improvement per quarter | Qualitative feedback |

## Appendix: Stakeholder Quotes Reference

Nick Luciano:

"We'd ideally get live streaming, but currently we just upload the data after-hours due to cost."

"The public should generally not know when the reactor is at power."

"The calendar is used to schedule time. It is not a reflection of what actually happened."

"[Xenon] cannot be measured directly, but can be correlated with critical rod heights."

"[Fuel burnup heatmap] would be great."

Jim (TJ):

"We can ascertain a baseline fuel 'burn up' amount when starting up from a long period of shut down."

"I am not sure what insight could be achieved by counting the number of log entries."

"Add a 'watch type' to all entries and then gather what operations we normally perform at what hour."

"A gap would mean that this :30 minute check was not performed when operating."

"Export to PDF would work, but a simple text file for archive and future proof would also work."

Document Status: Ready for technical review and Superset implementation planning